# iMHEA Data Processor — User Manual

Version 1.0 · July 2026

Table of Contents

# 1. Introduction

The iMHEA Data Processor converts raw sensor records from headwater catchment monitoring — tipping-bucket rain gauge events and water-level or discharge logger files — into quality-controlled rainfall and streamflow series at high, hourly, and daily resolution, and computes 59 hydrological and 13 climate indices describing water yield, regulation, flashiness, frequency, duration, timing, and rainfall intensity.

The methods implement Ochoa-Tocachi et al. (2018), High-resolution hydrometeorological data from a network of headwater catchments in the tropical Andes, Scientific Data 5:180080. The software is a validated Python translation of the original iMHEA MATLAB scripts: streamflow and baseflow outputs reproduce the published dataset exactly, and every known defect of the original code has been fixed (a compatibility mode can reproduce the legacy behaviour; see section 11).

Citing. If you use results from this software in a publication, cite the paper above.

# 2. Preparing your data

## 2.1 Raw file format

The software reads the standard iMHEA raw CSV layout. One file per sensor:

Rain gauge files (tipping-bucket events):

Date,Event mm,Flag
24/05/2001 00:00:01,0,I
24/05/2001 00:41:23,0.2,
03/04/2004 00:00:01,,V

Water level / discharge files:

Date,Level cm,Flow l/s,Flag
17/07/2012 14:00:00,8.1000,2.5582,"D,P"

Rules:

- Dates in dd/mm/yyyy HH:MM:SS, sorted ascending.

- Decimal point (.), comma-separated columns.

- Flags: I = logger launched, D = data downloaded, X = erroneous tip removed, P = anomalous intensity, V = data gap. A V row with an empty value marks the start of a gap — this is how the software knows data are missing rather than dry/zero.

- Encoding UTF-8 (a byte-order mark is tolerated); line endings may be Windows, Unix, or legacy Mac.

- Rainfall values are millimetres per tip; level in centimetres; flow in litres per second.

A machine-readable version of this specification is included as docs/DATA_FORMAT.md.

## 2.2 What you need to know about each catchment

| Parameter | Meaning | Typical iMHEA values |
| --- | --- | --- |
| Area | catchment area draining to the weir [km²] | 0.5 – 16 |
| Bucket | rain gauge resolution [mm per tip] | 0.1, 0.2, or 0.254 |
| Weir geometry | V-notch height a [m], rectangular width b [m] | 0.30 / site-specific |

# 3. Getting started

On launch, the start screen offers three paths:

- Process my data — creates an empty project to which you add your own catchments and files.

- Explore the iMHEA network — asks for the folder containing iMHEA_raw/ and loads the complete 2018 network configuration (28 stations, 12 pairs) ready to run.

- Open project… — reopens a saved .imhea project file.

A project stores your catchment configurations, file locations, and options. Save it from the toolbar (Save project); results are recomputed on demand and are not stored in the project file.

The toolbar also holds Run pipeline, Export, and the EN/ES language switch.

# 4. Configuring a catchment

Select a catchment in the sidebar and open its Setup tab.

- Enter the code (e.g. MYC_01), area, and bucket size.

- Under Discharge, add the level/flow file(s). If the file has a flow column it is used directly; choose Level → rating curve… to convert water level instead (section 4.1). Rain-only stations simply leave discharge empty.

- Under Rain gauges, add one file per gauge. Multiple gauges are cross-checked and averaged automatically (section 5).

- Each file shows a summary line — row count, time step, detected columns, period. Check these before running.

## 4.1 Rating curve editor

For stations recording water level only, the discharge is computed with the compound sharp-crested weir equation (90° V-notch inside a rectangular section):

- Within the notch (h ≤ a): Q = C1·h^e1

- Above it: Q = C1·(h^e1 − (h−a)^e1) + C2·b·(h−a)^e2

The editor lets you set a, b and the four coefficients (defaults: C1 = 1.37, e1 = 2.5, C2 = 1.77, e2 = 1.5) and shows the resulting curve live. Check b against your weir drawings — it is site-specific.

# 5. Running the pipeline

Press Run pipeline with a catchment selected. Processing runs in the background; messages stream into the Log tab. What happens, in order:

- Depuration — logger bounce tips arriving ≤ 1.1 s apart are removed.

- Grid selection — the working resolution is the median time step of the discharge record (5 min typical; rain-only stations use 5 min).

- Rainfall disaggregation — tip events become a continuous intensity series via cubic-spline interpolation of the cumulative curve (Sadler & Busscher 1989; Wang et al. 2008), preserving total volume; storms are separated at 0.2 mm/h, intensities capped at 127 mm/h, single tips spread at 3 mm/h.

- Gauge cross-filling — every pair of gauges is compared by double-mass analysis; gaps are filled proportionally only when the correlation R ≥ 0.99. Catchment rainfall is the average of the filled series.

- Discharge averaging to the grid; normalisation by area.

- Aggregation to hourly and daily products; baseflow separation (UKIH smoothed-minima and Chapman two-parameter filter).

- Indices — 59 hydrological + 13 climate indices (section 8).

The status bar reports the runtime (a few seconds per catchment typically).

# 6. Results

The Results tab shows summary cards (period, runoff ratio, baseflow index, gap percentage) and one of six figures, at daily, hourly or high resolution:

| Figure | Shows |
| --- | --- |
| Hydrograph | inverted rainfall bars over the flow series with shaded baseflow |
| Flow duration curve | percentage of time each flow is exceeded (log scale) |
| Intensity-duration curve | maximum rainfall intensity vs duration, 5 min – 2 days |
| Monthly regime | precipitation and runoff climatology, mm/month |
| Data coverage | timeline of valid data and gaps per variable |
| Double-mass curve | cumulative rainfall vs cumulative runoff; breaks in slope reveal changes in behaviour or instrumentation |

Figures are interactive (zoom, pan) and can be saved as PNG/PDF/SVG from the toolbar above the plot.

# 7. Gap-filling report

The Gap-filling report button (Results tab) lists every double-mass comparison of the last run: gauge pair, whether filling was applied, the correlation R, the slope M, and how many intervals were filled in each series. Use it to document quality control and to detect gauges that drift apart (R below 0.99 means no filling was applied — investigate why).

# 8. The indices

The Indices tab lists all values; Copy table and Save CSV… export them. Indices follow Olden & Poff (2003) nomenclature where applicable. Qxx denotes the flow exceeded xx% of the time; percentiles use Gringorten plotting positions. All flow indices are computed from daily mean flow in l/s/km²; rainfall intensity indices from the 5-minute grid.

## 8.1 Hydrological indices (59)

| # | Code | Description | Units |
| --- | --- | --- | --- |
| 1 | QDMIN | Minimum daily flow | l/s/km² |
| 2 | Q95 | Flow exceeded 95% of the time (low flow) | l/s/km² |
| 3 | DAYQ0 | Days with zero flow per year | day |
| 4 | PQ0 | Proportion of days with zero flow | – |
| 5 | QMDRY | Mean daily flow of the driest month | l/s/km² |
| 6 | QDMAX | Maximum daily flow | l/s/km² |
| 7 | Q10 | Flow exceeded 10% of the time (high flow) | l/s/km² |
| 8 | QDMY | Annual mean daily flow | l/s/km² |
| 9 | QDML | Long-term mean daily flow | l/s/km² |
| 10 | Q50 | Median flow from the FDC | l/s/km² |
| 11 | BFI1 | Baseflow index, UKIH smoothed-minima method | – |
| 12 | K1 | Recession constant, UKIH method | – |
| 13 | BFI2 | Baseflow index, Chapman 2-parameter filter | – |
| 14 | K2 | Recession constant, Chapman filter | – |
| 15 | RANGE | Discharge range QDMAX/QDMIN | – |
| 16 | R2FDC | Slope of the FDC between 33% and 66% exceedance | – |
| 17 | IRH | Hydrological regulation index | – |
| 18 | RBI1 | Richards-Baker flashiness index (annual) | – |
| 19 | RBI2 | Richards-Baker flashiness index (seasonal) | – |
| 20 | DRYQMEAN | Driest-month flow / mean monthly flow | – |
| 21 | DRYQWET | Driest-month flow / wettest-month flow | – |
| 22 | SINDQ | Streamflow seasonality index | – |
| 23 | QYEAR | Average annual discharge | mm |
| 24 | RRa | Annual runoff ratio QYEAR/PYEAR | – |
| 25 | RRm | Monthly runoff ratio | – |
| 26 | RRl | Long-term runoff ratio | – |
| 27 | MA5 | Skewness of daily flows: mean/median | – |
| 28 | MA41 | Mean annual runoff | l/s/km² |
| 29 | MA3 | Coefficient of variation of daily flows | – |
| 30 | MA11 | (Q25 − Q75) / median | – |
| 31 | ML17 | 7-day minimum flow / mean annual flow | – |
| 32 | ML21 | CV of 30-day minimum flows | – |
| 33 | ML18 | CV of 7-day minimum flows | – |
| 34 | MH16 | High flow: Q10 / median | – |
| 35 | MH14 | Median 30-day maximum flow / median | – |
| 36 | MH22 | Mean high-flow volume above 3× median / median | day |
| 37 | MH27 | Mean peak above Q25 / median | – |
| 38 | FL3 | Low pulses below 5% of mean daily flow | 1/yr |
| 39 | FL2 | CV of annual low-pulse counts (FL1) | – |
| 40 | FL1 | Low pulses below Q75 | 1/yr |
| 41 | FH3 | High pulses above 3× median daily flow | 1/yr |
| 42 | FH6 | High-flow events above 3× median monthly flow | 1/yr |
| 43 | FH7 | High-flow events above 7× median monthly flow | 1/yr |
| 44 | FH2 | CV of annual high-pulse counts (FH1) | – |
| 45 | FH1 | High pulses above Q25 | 1/yr |
| 46 | DL17 | CV of low-pulse durations (DL16) | – |
| 47 | DL16 | Mean low-pulse duration below Q75 | day |
| 48 | DL13 | Mean 30-day minimum flow / median | – |
| 49 | DH13 | Mean 30-day maximum flow / median | – |
| 50 | DH16 | CV of high-pulse durations (DH15) | – |
| 51 | DH20 | Mean high-pulse duration above median/0.75 | day |
| 52 | DH15 | Mean high-pulse duration above Q25 | day |
| 53 | TH3 | Longest period with no flood (above Q10), fraction of year | – |
| 54 | TL2 | CV of TL1 | – |
| 55 | TL1 | Median Julian day of the annual 1-day minimum | – |
| 56 | RA8 | Rate of flow reversals between days | 1/day |
| 57 | RA5 | Fraction of days with rising flow | – |
| 58 | RA6 | Median rise rate, log space | – |
| 59 | RA7 | Median fall rate, log space | – |

## 8.2 Climate indices (13)

| # | Code | Description | Units |
| --- | --- | --- | --- |
| 1 | PYEAR | Average annual precipitation | mm |
| 2 | DAYP0 | Days with zero precipitation per year | day |
| 3 | PP0 | Proportion of days with zero precipitation | – |
| 4 | PMDRY | Precipitation of the driest month | mm |
| 5 | SINDX | Seasonality index (0 none – 1 extreme) | – |
| 6 | PVAR | Coefficient of variation of daily precipitation | – |
| 7 | RMED1D | Median annual maximum 1-day precipitation | mm |
| 8 | RMED2D | Median annual maximum 2-day precipitation | mm |
| 9 | RMED1H | Median annual maximum 1-hour precipitation | mm |
| 10 | iMAX1D | Maximum 1-day intensity | mm/h |
| 11 | iMAX2D | Maximum 2-day intensity | mm/h |
| 12 | iMAX1H | Maximum 1-hour intensity | mm/h |
| 13 | iMAX15M | Maximum 15-minute intensity | mm/h |

(The published dataset lists two further rows, ETYEAR and RPPE, derived from external evapotranspiration data; they are not computed by this software.)

# 9. Paired catchments

The iMHEA design compares paired catchments (e.g. conserved vs intervened). Create a pair with + Add pair after configuring both catchments. Running a pair:

- resamples both to the coarser common resolution;

- cross-fills rainfall between the catchments by double-mass analysis (discharge is never filled);

- produces combined high-resolution/hourly/daily tables and side-by-side indices, with comparison figures (overlaid hydrographs, FDCs, IDCs).

# 10. Exporting

Export writes, to a folder you choose:

- iMHEA_<CODE>_HRes_processed.csv — high-resolution [Date, Rainfall, Flow]

- iMHEA_<CODE>_1hr_processed.csv — hourly

- iMHEA_<CODE>_1day_processed.csv — daily, including baseflow

- for pairs, the corresponding _01/_02 files in the published iMHEA format (dates dd/mm/yyyy HH:MM:SS, fixed-width values, NaN for gaps).

Indices export from the Indices tab (CSV or clipboard).

# 11. MATLAB compatibility mode

During translation, nine defects were found in the original MATLAB code (documented in docs/CODE_REVIEW.md and quantified in docs/VALIDATION.md). By default this software computes the corrected values. Ticking MATLAB-compatible mode (Setup tab) reproduces the legacy behaviour where feasible — useful when comparing against the 2018 published dataset. The differences that matter most:

- BFI1/K1: published values are inflated for records with data gaps (gap blocks entered the baseflow line as turning points).

- TL1/TL2: published timing values are unreliable after the first year of each record (day-of-year indexing defect).

- FH7: published values are double-normalised (too small by roughly the number of years).

# 12. Command-line reference

For batch work, the imhea command (requires the Python package):

imhea process --code MYC_01 --area 2.63 --bucket 0.2 \
    --flow flow.csv --gauge rg1.csv --gauge rg2.csv --out ./processed
imhea process --code RAIN_01 --bucket 0.1 --gauge rg.csv --out ./out
imhea pair --hres1 A_HRes.csv --hres2 B_HRes.csv --site XYZ --out ./out
imhea network --data-root ./Scripts --out ./valout
imhea --version

--level-to-flow (with --weir A B and --coeff C1 E1 C2 E2) converts a level column through the rating curve; --matlab-compat enables the compatibility mode.

# 13. Frequently asked questions

How much data is needed for meaningful indices? At least one full year; the software computes indices for any record length without warning, so interpret short-record results cautiously (annual statistics treat partial years as full ones, as in the original methodology).

Why does my rainfall total change slightly after processing? Depuration removes physically impossible double tips (≤ 1.1 s apart); the log reports the removed volume. Disaggregation itself conserves volume.

Gaps appear where I expect data. Gaps propagate from V-flag marker rows in the raw files. Check the Data coverage figure and the raw files’ flag column.

Can I process data from non-iMHEA loggers? Yes — convert to the raw CSV layout of section 2.1. Only the date format, column layout, and the V gap convention are mandatory.

Where are my project’s results stored? Results live in memory and are recomputed on demand; exported CSVs and the .imhea project file are the persistent artefacts.

# 14. References

- Ochoa-Tocachi, B.F., et al. (2018). High-resolution hydrometeorological data from a network of headwater catchments in the tropical Andes. Scientific Data 5, 180080.

- Gustard, A., Bullock, A., Dixon, J.M. (1992). Low flow estimation in the United Kingdom. Institute of Hydrology Report 108.

- Chapman, T. (1999). A comparison of algorithms for stream flow recession and baseflow separation. Hydrological Processes 13.

- Olden, J.D., Poff, N.L. (2003). Redundancy and the choice of hydrologic indices for characterizing streamflow regimes. River Research and Applications 19.

- Sadler, E.J., Busscher, W.J. (1989). High-intensity rainfall rate determination from tipping-bucket rain gauge data. Agronomy Journal 81.

- Wang, J., et al. (2008). A new treatment of depth-dependent errors in tipping-bucket rain gauge measurements. J. Atmos. Oceanic Technol. 25.